Document Purpose

This control narrative outlines an advanced DCS/PLC system for Lube Oil Blending Plant (LOBP) operations, integrating AI for recipe optimization, resource management, predictive quality, safety, and Industry 4.0 features. Prepared as an EPC deliverable, it details sequences, interlocks, alarms, and HMI for commissioning, FAT/SAT, and operator training.

Process Overview

Automates batch/inline blending of base oils, additives, and modifiers across tank farms, pumps, pipelines, mixers, and analyzers. DCS collects real-time data (flow, density, viscosity, temperature, levels); AI submodule processes via ML models for dynamic control. PLC executes valve/pump actions; HMI provides dashboards, simulations, and voice interfaces.

Core Control Functionalities (initial phase of implementation – in 2 phases prioritized)

- Recipe Optimization: AI dynamically adjusts ratios using neural networks trained on historical/sensor data to match specs (viscosity, flash point, pour point).

- Predictive Quality Control: Continuous monitoring predicts properties pre-blend; auto-corrects deviations, holds if off-spec risk >5%.

- Pump Sequencing & Occupancy: Sequences pumps by tank levels/pipeline occupancy (e.g., additives first); interlocks at 80%/20% thresholds.

- Dynamic Rerouting/Rescheduling: ML algorithms reroute flows, reschedule queues to minimize idle/changeovers.

- Adaptive Learning: Refines models from lab feedback; throttles energy for 15-20% savings.

- Multi-Blend Parallelism: Concurrent 2-3 blends with anti-contamination interlocks.

- Raw Material Tank Optimization: Forecasts depletion, reallocates heels/FIFO, swaps for segregation using nonlinear programming.

- Supply Chain Integration: Pre-schedules via supplier APIs for delivery delays.

Advanced Features (final phase of implementation)

- Safety & Hazop: Predicts risks (over-pressure, incompatibility); real-time API/REACH compliance logging.

- Energy/Waste Recovery: Repurposes mixer heat for pre-heating; evaporation compensation.

- Maintenance & Twins: Predictive wear via vibration data; digital twins for what-if scenarios.

- Operator Aids: AR overlays, natural language queries, explainable anomalies.

- Scalability: Demand forecasting, cloud fleet optimization, multi-site load balancing.

- Innovation: Generative AI for new recipes (e.g., bio-based EV oils).

- Sustainability: Carbon footprint tracking (Scope 1-3), ESG reports. (future trends and possibly final phase of implementation)

- Resilience: Cyber anomaly detection, blockchain traceability. (future trends)

- Robotics: Product sampling with real-time analyzer feedback. (future trends)

Control Sequences

- Initiation: Operator selects recipe; AI validates inventory/occupancy, proposes sequence.

- Execution: DCS sequences pumps/valves; AI monitors/adjusts, predicts/reroutes.

- Completion: Auto-transfer, lab verification, report with simulations.

- Abnormal: Deviation → hold/alarm; occupancy breach → shutdown; faults → reroute/notify.

Alarms & Interlocks

| Trigger | Action | Priority |
| --- | --- | --- |
| Off-spec risk >5% | Hold batch, suggest correction | High |
| Occupancy 90%/20% | Block starts, AI reallocation | Critical |
| Pump fault/Anomaly | Reroute to backup, maintenance log | Medium |
| Cyber threat | Isolate node, fail-safe mode | Critical |
| Non-compliance | Halt, audit export | High |

HMI & Reporting

- Dashboards: Real-time KPIs (yield, energy/liter, throughput), AI recommendations, queues.

- Simulations: Offline recipe/schedule testing.

- Analytics: Trends, benchmarks, ESG/carbon reports.

Quantified Benefits – as example – numbers to be determined

| Category | Improvement | Business Impact |
| --- | --- | --- |
| Cost | 20-35% less additives/waste | Raw material savings |
| Efficiency | 25% throughput, faster cycles | Higher utilization |
| Quality | <1% off-spec | Minimal re-blends/testing |
| Energy/Sustainability | 15-20% reduction | Net-zero alignment |
| Operations | Adaptive, resilient | Downtime <5% |